## PORTABLE LADDERS

- Are all ladders maintained in good condition, joints between steps and side rails tight, all hardware and fittings securely attached and moveable parts operating freely without binding or undue play?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are non-slip safety feet provided on each ladder?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are non-slip safety feet provided on each metal or rung ladder?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are ladder rungs and steps free of grease and oil?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is it prohibited to place a ladder in front of doors opening toward the ladder except when the door is blocked open, locked or guarded?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is it prohibited to place ladders on boxes, barrels, or other unstable bases to obtain additional height?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are employees instructed to face the ladder when ascending or descending?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are employees prohibited from using ladders that are broken, missing steps, rungs, or cleats, broken side rails or other faulty equipment?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are employees instructed not to use the top step of ordinary stepladders as a step?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- When portable rung ladders are used to gain access to elevated platforms, roofs, etc., does the ladder always extend at least 3 feet above the elevated surface?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is it required that when portable rung or cleat type ladders are used, the base is so placed that slipping will not occur, or it is lashed or otherwise held in place?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are portable metal ladders legibly marked with signs reading "CAUTION - Do Not Use Around Electrical Equipment" or equivalent wording?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are employees prohibited from using ladders as guys, braces, skids, gin poles, or for other than their intended purposes?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are employees instructed to only adjust extension ladders while standing at a base (not while standing on the ladder or from a position above the ladder)?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are metal ladders inspected for damage?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are the rungs of ladders uniformly spaced at 12 inches, center to center?

Action to be Taken						Assigned To:

_____________________________________   ____________________________