## MACHINE GUARDING

- Is there a training program to instruct employees on safe methods of machine operation?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is there adequate supervision to ensure that employees are following safe machine operating procedures?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is there a regular program of safety inspection of machinery and equipment?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is all machinery and equipment kept clean and properly maintained?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is sufficient clearance provided around and between machines to allow for safe operations, set up and servicing, material handling and waste removal?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is equipment and machinery securely placed and anchored, when necessary to prevent tipping or other movement that could result in personal injury?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Is there a power shut-off switch within easy reach of the operator's position at each machine?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Can electric power to each machine be locked out for maintenance, repair, or security?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are the noncurrent-carrying metal parts of electrically operated machines bonded and grounded?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are foot-operated-switches guarded or arranged to prevent accidental actuation by personnel or falling objects?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are manually operated valves and switches controlling the operation of equipment and machines clearly identified and readily accessible?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are all emergency stop buttons colored red?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are all pulleys and belts that are within 7 feet of the floor or working level properly guarded?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are all moving chains and gears properly guarded?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are splashguards mounted on machines that use metal working fluids to prevent the fluids or aerosols from reaching employees?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are methods provided to protect the operator and other employees in the machine area from hazards created at the point of operation, ingoing nip points, rotating parts, flying chips, and sparks?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are machinery guards secure and so arranged that they do not offer a hazard in their use?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- If special handtools are used for placing and removing material, do they protect the operator's hand?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are revolving drums, barrels, and containers required to be guarded by an enclosure that is interlocked with the drive mechanism, so that revolution cannot occur unless the guard enclosures is in place, so guarded?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Do arbors and mandrels have firm and secure bearings and are they free from play?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are provisions made to prevent machines from automatically starting when power is restored after a power failure or shutdown?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are machines constructed so as to be free from excessive vibration when the largest size tool is mounted and run at full speed?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- If machinery is cleaned with compressed air, is air pressure controlled and personal protective equipment or other safeguards utilized to protect operators and other workers from eye and body injury?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are fan blades protected with a guard having openings no larger than 1/2 inch, when operating within 7 feet of the floor?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are saws used for ripping, guarded by a hood, which automatically adjusts itself to the thickness of the material?

Action to be Taken						Assigned To:

_____________________________________   ____________________________

- Are radial arm saws guarded so that the guard will automatically adjust itself to the thickness of the stock and remain in contact with the stock?

Action to be Taken						Assigned To:

_____________________________________   ____________________________